*****************

*  Preliminary  *

*****************

Installation Guide of CAMAC Device Driver/Library on SPARC

7-Jun-1993

Version 1.22

Yasuo TAKEUCHI

Dept. of Physics, Tokyo Institute of Technology (T.I.T.)

Copyright reserved.

0. Overview

This CAMAC driver/library has following features.

1. It can work on SPARC/SunOS 4.1.X with SBus-VME adapter.

(Solflower SF-VME, etc.)

2. All CAMAC functions are callable from C and FORTRAN programs.

3. These functions (camlib) are compatible with those on VAX/VMS,

DEC station/ULTRIX developed by KEK.

4. Provide the CAMAC list processing in the UNIX kernel mode.

5. All driver source is written in C language.

This document describes following topics briefly.

1. How to get the CAMAC driver source.

2. Setup of hardware (K2917, SF-VME)

3. Installation of software (Solflower's driver, CAMAC driver)

4. How to test the SPARC-VME-CAMAC system.

5. E-mail address, etc.

1. Getting Driver Source

You can get the driver source by using "anonymous FTP".

FTP host: onlnews.kek.jp  (130.87.97.1)

bangkok.ssc.gov (134.3.30.11)

PROMPT% ftp onlnews.kek.jp

Name(onlnews.kek.jp:xxxx): anonymous

Password: <your E-mail address>

ftp> cd pub/kek/camac

ftp> get Install-guide.sparc.txt	<= this document

ftp> get camexe_camasm.txt		<= documents

ftp> binary

ftp> get camac.sparc-1.22b.tar.Z	<= driver source

ftp> get camlib.docs-0.81.tar.Z		<= documents

ftp> bye

In above example, "PROMPT%" represents your prompt. Next,

you should extract these archives.

PROMPT% zcat camac.sparc-1.22b.tar.Z | tar xvf -

PROMPT% cd vme

PROMPT% zcat ../camlib.docs-0.81.tar.Z | tar xvf -

2. Setup of Hardware

1. Perform shutdown of your SPARCstation, then turn off the power.

PROMPT% su

PROMPT# shoutdown -h now

2. Install Solflower's SBus board to suitable SBus slot

except SBus slot 3. The SBus board is used as default setting.

3. Set jumper switch of K2917. Change the DMA LO/HI setting(default)

to DMA HI/LO. Please refer K2917's manual.

4. Insert the K2917 VME board into SF-VME's VME rack. You can use

any slot of VME. Before insertion, You must remove the 5

jumper pins at left side of upper 96pin connector in the VME

backplane.

5. Connect VME with SPARC by using SF-VME's cable.

6. Turn on the power. (VME rack, CPU) Up to here, you can install

the driver, so you can leave following steps later.

7. Setup K3922 according to the K3922's manual.

8. Connect K3922 with K2917 by the suitable cable. This cable

isn't included K2917 or K3922 products, so you may have to

assemble it by yourself.

9. Insert K3922 into the CAMAC crate, then turn power on.

3. Installing of CAMAC Device Driver and Library

1. Load SF-VME's driver from the floppy disk to work directory.

PROMPT% cd ~/vme/sfvme

(Insert FD)

PROMPT% tar xvf /dev/rfd0

PROMPT% eject

2. Copy SFVME's files according to SFVME's manual.

Before copying you should make backup of original files.

PROMPT% cat install.sh		<= check!

PROMPT% cat Makefile		<= check!

PROMPT% su

PROMPT# make backup

PROMPT# sh install.sh

PROMPT# exit

3. Make symbolic link of the header files of SFVME.

PROMPT% su

PROMPT# make header

PROMPT# exit

4. Make the device entry points of SF-VME. Please refer SF-VME's

manual and examples of modification. (~/vme/camac/titech/*)

PROMPT% more /sys/sun/conf.c.original <= check last number

of devices

PROMPT% vi SFVME.MAKEDEV	<= modify "sfvmemajor=104"

if needed

PROMPT% su

PROMPT# cd /dev

PROMPT# sh /home/xxxxxxxx/vme/sfvme/SFVME.MAKEDEV

PROMPT# chmod 666 sfvme*

PROMPT# ls -al sfvme*		<- check!

PROMPT# exit

5. Next, install CAMAC driver files, then recompile the kernel.

PROMPT% cd ~/vme/camac/dev

6. Copy original files to the current directory.

PROMPT% make newfiles

7. Modify ./CAMAC ./conf.c ./files. Please refer our modification

examples and SF-VME's manual.

PROMPT% diff CAMAC  ../titech/4.1.1/org/CAMAC

PROMPT% diff CAMAC  ../titech/4.1.1/mod/CAMAC

PROMPT% diff conf.c ../titech/4.1.1/org/conf.c

PROMPT% diff files  ../titech/4.1.1/org/files

(etc..)

PROMPT% vi CAMAC	<= modify 1 point

PROMPT% vi conf.c	<= modify 2 points

PROMPT% vi files	<= modify last several lines

8. Make the CAMAC driver entry point. The device number of

CAMAC is next to "sfvmemajor".

PROMPT% vi Makefile		<- modify "105" at last line

PROMPT% su

PROMPT# make node

PROMPT# exit

9. Keep current kernel, then recompile new kernel with CAMAC driver.

Then reboot the system.

PROMPT% su

PROMPT# df			<- check capacity of /

PROMPT# mv /vmunix /vmunix.original

PROMPT# make kernel

PROMPT# fastboot

10. Check boot messages. Are there following messages?

vme0 at SBus slot 1 0x7ff000 ...

cc0 at vme16d16 0xff00 vec 0xff ...

CAMAC device driver V1.22b, 1991-1993 by Y.TAKEUCHI (T.I.T.)

11. Compile the CAMAC library(camlib), then install them.

PROMPT% cd ~/vme/camac

PROMPT% vi Makefile		<- modify DISTDIR, if needed

PROMPT% make library

PROMPT% su

PROMPT# make install

PROMPT# exit

12. Compile test programs.

PROMPT% make sample

4. Test CAMAC action etc.

In this section, an example of examination step is described.

The programs used for the test are included in ~/vme/tools or

~/vme/camac/examples.

4.1.	Test of reading SF-VME's register. (~/vme/tools/sreg1.c)

This program consists from only SunOS's function. So this test

proves which SF-VME is successfully installed or not.

From the boot message, it is known which slot is used by

Solflower's SBus board. (1 at above example message) You should

modify the access privilege of that slot.

PROMPT% su

PROMPT# chmod 666 /dev/sbus1

PROMPT# exit

Next, modify sbus_slot.h, then compile sreg1.c.

PROMPT% cd ~/vme/tools

PROMPT% vi sbus_slot.h		<- comment out proper line

PROMPT% make sreg1

Then execute sreg1.

PROMPT% sreg1

4.2.	Test of reading K2917's register. (~/vme/tools/kreg1.c)

This program consists from only SunOS's and SF-VME's function.

This test proves which SF-VME's driver and K2917 is successfully

installed or not.

PROMPT% cd ~/vme/tools

PROMPT% make kreg1

PROMPT% kreg1

4.3.	Test of CAMAC single action. (~/vme/camac/examples/cam1.c)

This program consists from SunOS's and CAMAC's function.

Before this test, connect K3922 with K2917, then turn CAMAC

crate's power on.

PROMPT% cd ~/vme/camac/examples

PROMPT% make cam1

PROMPT% cam1

4.4.	Test of CAMAC block transfer. (~/vme/camac/fortran/cam2.f)

This program consists from SunOS's and CAMAC's function.

From this examination, it is proved which the DMA works well

or not.

PROMPT% cd ~/vme/camac/fortran

PROMPT% make cam2

PROMPT% cam2

4.5.	Test of LAM interruption. (~/vme/camac/examples/cam3.c)

This program consists from SunOS's and CAMAC's function, and

uses SF-VME's interrupt handling function internally.

The program assumes CAMAC LAM source at N=11. Please modify

for your environment.

PROMPT% cd ~/vme/camac/examples

PROMPT% make cam3

PROMPT% cam3

4.6.	Test of CAMAC list processing. (~/vme/camac/examples/cam4.c)

This test check the action of CAMAC list processing.

Please modify cam4.asm, if you needed.

(please refer camexe_camasm.txt)

PROMPT% cd ~/vme/camac/examples

PROMPT% make cam4

PROMPT% cam4

5.	References

1. Y.Takeuchi et al., Development of Data Acquisition System

using RISC/UNIX(TM) Workstation, Nucl. Instrum. Meth.

A328(1993)526-534

2. Y.YASU and M.NOMACHI, CAMAC Device Driver on VAX/VMS version 1.0,

KEK Internal 90-2, 1990

3. M.Nomachi, CAMEXE (a virtual machine which execute CAMAC operations)

and CAMASM (a macro assembler for CAMEXE machine code),

KEK on-line group, 1987

(camasm_camexe.txt)

6.	Address for bug-reports etc.

Please send bug-reports, requests, impression and so on

to following address.

*******************************************************************************

Yasuo TAKEUCHI

Watanabe-Tanimori group,

Dept. of Physics,

Tokyo Inst. of Technology,

2-12-1 O-okayama, Meguro-ku, Tokyo 152, JAPAN

FAX		-81-3-3720-9083

E-mail		take@kekvax.kek.jp

KEKVAX::TAKE

TAKE@JPNKEKVX

*******************************************************************************